  Multicast Mesh Creation:

- CAMP creates a shared mesh for each multicast group, which helps maintain connectivity among multicast users, even when nodes move.

- The mesh is essentially a network of interconnected nodes that support multicast communication.

  Control Traffic and Core Nodes:

- CAMP uses core nodes to handle control traffic related to joining and maintaining multicast groups.

- Core nodes are crucial for managing group membership and facilitating communication within the multicast group.

  Router Classification:

- Simplex Mode: A router in this mode only sends multicast traffic received from specific nodes to the rest of the group. It does not forward multicast packets from other nodes.

- Duplex Mode: A router in this mode forwards multicast packets for the group and participates actively in the multicast mesh.

- Non-Member: A router that does not participate in the multicast mesh and does not need to forward multicast traffic.

  Joining and Leaving Groups:

- Receiver-Initiated Method: Routers join a multicast group by announcing their membership. If a router has multiple neighbors that are already members, it updates its routing table and announces itself to these neighbors.

- If no direct neighbors are members, the router sends a join request to a core node or uses an expanding ring search to find a member.

- Routers can leave a multicast group if they no longer have hosts that need the group or if they are not required for efficient packet dissemination.

  Maintaining Connectivity:

- CAMP ensures that all reverse shortest paths between a source and recipients are maintained in the mesh.

- Receiver nodes periodically check if they are receiving data packets via the shortest path. If not, they send a HEARTBEAT message to their successor, which triggers a PUSH JOIN (PJ) message. This process forces non-member nodes in the path to join the mesh.

  Advantages and Disadvantages:

- Advantages:

  - CAMP avoids flooding, which limits the spread of multicast join requests to mesh members only.

- Disadvantages:

  - It depends on an underlying unicast routing protocol to ensure that distances to all destinations are accurate and updated within a finite time frame.

AMRoute Protocol Overview

- Core Nodes and Their Role:

  - Non-Core Nodes:

    - Serve as passive agents that only respond to messages from other core nodes.

    - The selection of a logical core is dynamic and can change based on network dynamics and group membership.

- Mesh Creation:

  - Initial Process:

    - Each member node starts by identifying itself as a core and broadcasts JOIN_REQ packets.

    - These packets use increasing Time-to-Live (TTL) values to discover other core nodes in the network.

  - Core Response:

    - When a core node receives a JOIN_REQ from another core node in a different mesh for the same group, it replies with a JOIN_ACK.

    - This creates a bi-directional tunnel between the two cores and one of them is selected as the core after the mesh merger.

- Tree Creation:

  - Initiation:

    - Once the mesh is established, the core node initiates the tree creation process.

    - The core node sends periodic TREE_CREATE messages along all links in its mesh.

  - Message Forwarding:

    - TREE_CREATE messages are sent using unicast tunnels only to group members.

    - Group members receiving a non-duplicate TREE_CREATE message will forward it to all mesh links except the incoming one.

    - They mark the incoming and outgoing links as tree links.

  - Handling Unused Links:

    - If a link is not going to be used as part of the tree, the TREE_CREATE message is discarded, and a TREE_CREATE_NAK message is sent back to the incoming link.

- Leaving a Group:

  - Process:

    - A member node wishing to leave the group sends a JOIN_NAK message to its neighboring nodes.

- Virtual Mesh Links:

  - Function:

    - Virtual mesh links are used to establish and maintain the multicast tree.

    - This helps keep the multicast delivery tree consistent even with changes in network topology, as long as routes between core nodes and tree members exist via mesh links.

- Disadvantages:

  - Temporary Loops and Non-Optimal Trees:

    - AMRoute may experience temporary loops and create non-optimal multicast trees due to network mobility.

The Multicast Core-Extraction Distributed Ad Hoc Routing (MCEDAR) protocol is designed to enhance multicast routing by combining the strengths of tree-based and mesh-based forwarding protocols. Here’s a detailed breakdown of its components and operations:

MCEDAR Protocol Overview

- Objective:

  - MCEDAR aims to integrate the efficiency of tree-based forwarding with the robustness of mesh-based protocols.

- Combining Approaches:

  - Tree-Based Forwarding:

    - Creates a source-based forwarding tree on a mesh to ensure efficient data delivery.

  - Mesh-Based Protocols:

    - Provides robustness by using a mesh structure that can tolerate link failures.

- Forwarding Tree on a Mesh:

  - Source-Based Forwarding Tree:

    - Built on top of the mesh infrastructure.

    - Ensures that data forwarding occurs over the shortest paths (minimum height trees).

- Decoupling Control and Data Forwarding:

  - Control Infrastructure:

    - Separate from actual data forwarding to reduce control overhead.

  - Unicast Protocol:

    - The underlying unicast protocol, CEDAR, handles core broadcasting for multicasting.

- Core Nodes and Mesh Infrastructure:

  - Core Nodes:

    - Used for routing management and link state inspection.

  - Mesh Infrastructure (mgraph):

    - Cores form the mesh structure (referred to as mgraph) and use join IDs to perform join operations.

- Mesh Tolerance:

  - Link Breakage Tolerance:

    - MCEDAR can tolerate some link failures without needing reconfiguration, thanks to its mesh-based approach.

- Forwarding Mechanism:

  - Implicit Route-Based Forwarding Tree:

    - The mesh uses this mechanism to ensure packets travel the shortest distance.

  - Efficiency:

    - Achieved by creating a forwarding tree on the mesh.